# School of Engineering

B Tech CSE (Data Science), Capstone Project Weekly Progress Report

Semester: VII

Week No: 2	                              Date: From 31-08-2026 To 05-09-2026

| Department | Computer Science & Engineering (Data Science) |
| --- | --- |
| Names & USN of Team Members | Ayush Singh (ENG23DS0098) |
| Name of the Guide | Dr. Jobin Thomas |
| Project Title | A GPU-Accelerated Ray-Traced Urban Microclimate Simulation and AI Based Cooling Design Optimization Framework |

| Work Carried out by the team during the Week (in Bullet points only): Successfully downloaded Overture building footprints for Washington Square Park (~350 buildings), verified height attribute coverage (~85% of buildings have height data), and confirmed CRS is EPSG:4326; flagged buildings with missing heights for fallback estimation. Downloaded ERA5 reanalysis data for 15 July 2024 via CDS API and extracted variables at 18:00 UTC: 2m temperature (33.2°C), dewpoint temperature, 10m wind components (U10, V10), surface solar radiation downwards (SSRD), total sky radiation downwards (STRD), and clear-sky SSRD; converted units (K→°C, J/m²→W/m²), computed relative humidity (~42%) via Magnus formula, and computed pedestrian-height wind speed (~1.3 m/s at 1.1m) using logarithmic profile with z0=0.7. Downloaded SRTM 30m DEM tile for the study area, reprojected to EPSG:32618, resampled to 1m resolution via bilinear interpolation, and computed relative ground elevation (range ~3m across the study area, confirming flat terrain assumption). Implemented the harmonize.py module: reprojected building GeoDataFrame to UTM, defined the 1m target grid using rasterio.transform.from_bounds(), rasterized building footprints with heights onto the ground DEM (all_touched=False), and produced the unified DSM (NumPy array) along with buildings table, grid metadata (transform, bounds, resolution, CRS, shape), and pedestrian grid parameters. Visualized the DSM with matplotlib and verified buildings appear as elevated blocks on the ground surface at correct locations; confirmed elevation range (18–45m) is consistent with NYC mid-rise urban morphology. |  |
| --- | --- |
| Individual Contribution from the Student: |  |
| Ayush Singh | Downloaded, processed, and validated all three primary data sources (buildings, weather, terrain), implemented the data harmonization logic that unifies them into a single 1m UTM grid, and produced the first visual sanity check confirming the DSM accurately represents the study area geometry. |
| Work Planned for the Following Week: Build the pedestrian point grid at 1.1m height with walkable surface mask, and optionally construct 3D watertight building meshes for the future reference ray tracer. Implement the solar.py physics module computing solar altitude and azimuth; validate against NOAA calculator for multiple test cases. Implement the svf.py module with horizon ray-marching on the DSM; validate against analytical cases (flat ground SVF=1.0, deep canyon, isolated cube). |  |
| Major Bottlenecks (if any) Building height gap: ~15% of Overture buildings lack the height attribute; fallback estimation via num_floors * 3.0 or default 8m introduces uncertainty in DSM accuracy for those buildings; documented as a known limitation. SRTM 30m to 1m resampling does not add real terrain detail (acceptable for flat Washington Square Park, but must be flagged for future steeper terrain study areas where 30m source data would be insufficient). |  |

Percentage Work Completed:	22%

Signature of the Students:	Signature of the Guide