# School of Engineering

B Tech CSE(Data Science), Capstone  Project Weekly Progress Report

Semester: VII

Week No: 3	                           Date    : From 07-09-2026 To 12-09-2026

| Department | Computer Science & Engineering (Data Science) |
| --- | --- |
| Names & USN of Team Members | Ayush Singh (ENG23DS0098) |
| Name of the Guide | Dr. Jobin Thomas |
| Project Title | A GPU-Accelerated Ray-Traced Urban Microclimate Simulation and AI Based Cooling Design Optimization Framework |

| Work Carried out by the team during the Week (in Bullet points only): Created the pedestrian point grid (N=40,000 points, N×3 array at 1.1m height) aligned to 1m DSM pixel centers, with a walkable Boolean mask excluding points inside building footprints; verified point count matches 200×200m at 1m resolution and mask coverage matches building locations. Implemented `solar.py` module computing solar altitude and azimuth from latitude, longitude, date, and UTC hour using Cooper declination model and spherical trigonometry; validated for NYC on 15 July at 14:00 local (altitude=62.3°, azimuth=140.2°) against NOAA solar calculator; wrote `tests/test_solar.py` with multiple test cases. Implemented `svf.py` module computing Sky View Factor via horizon ray-marching in 360 azimuth directions on the DSM with 200m maximum search radius using Steyn cos² integration; validated flat open area SVF=0.98 (expected ~1.0), deep canyon SVF=0.14–0.35, and open park area SVF=0.82–0.95; wrote `tests/test_svf.py`. Implemented `shadows.py` module casting 2.5D shadows by checking whether each pedestrian point is above the shadow line from the nearest taller object in the sun direction; validated shadow direction (opposite to sun azimuth, north side of buildings for south-positioned sun) and shadow length (height/tan(altitude) = 11.5m for 20m building at 60° altitude); visualized shadow mask overlaid on DSM confirming correct placement; wrote `tests/test_shadows.py`. Implemented `radiation.py` computing direct solar flux (SSRD × cos(altitude) × atmospheric transmission × sunlit flag), diffuse solar flux (STRD_diffuse × SVF), downward longwave (ε_sky × σ × T_sky⁴), and upward longwave (ε_ground × σ × T_surface⁴); validated sunlit open area total flux ~300–600 W/m² and shaded canyon flux ~100–200 W/m²; wrote `tests/test_radiation.py`. Implemented `tmrt.py` converting total absorbed radiant flux to Mean Radiant Temperature via Stefan-Boltzmann law with human cylinder view factor weights (0.06 top, 0.06 bottom, 0.22 × 4 sides); validated sunlit open area Tmrt=52–68°C (expected >50°C) and shaded area Tmrt=28–42°C (expected <40°C); wrote `tests/test_tmrt.py`. |  |
| --- | --- |
| Individual Contribution from the Student: |  |
| Ayush Singh | Implemented and validated the complete core radiative physics chain (solar position → SVF → shadows → radiation fluxes → Mean Radiant Temperature) with individual unit tests for each module, establishing the scientifically grounded computation pipeline that converts urban geometry and weather into pedestrian-level thermal exposure maps; all four physics modules pass their tests and produce physically reasonable outputs for the Washington Square Park study area. |
| Work Planned for the Following Week: Implement `utci.py` module computing Universal Thermal Climate Index from Ta, Tmrt, pedestrian wind speed, and RH using pythermalcomfort; validate UTCI (sunlit) > UTCI (shaded) by large margin and against known UTCI reference ranges. Write `maps.py` visualization module producing five publication-quality PNG figures (DSM, shadow mask, SVF, Tmrt, UTCI) with colorbars, scale bars, north arrows, and statistics boxes. Write the end-to-end orchestrator `run_stage1.py` wiring all modules in sequence, add validation sanity checks, and produce the stage1 validation report. |  |
| Major Bottlenecks (if any) Stefan-Boltzmann calculation requires careful handling of Kelvin vs Celsius temperature scale; a unit error produces Tmrt values off by 273°C; added explicit assertions checking that Tmrt values fall within physically plausible range (0–100°C) before proceeding. Diffuse solar radiation estimation depends on cloud fraction derived from ERA5 SSRD/SSRD_clear ratio; for a single time step this ratio can be noisy; using a fixed conservative estimate of 0.1 cloud fraction for the hot summer day and documenting the assumption as a limitation. |  |

Percentage Work Completed:	27%

Signature of the Students:	Signature of the Guide